CPE301 – SPRING 2019

Design Assignment 2A TASK1

Student Name: yeeunLee

Student #:2001614222

Student Email: leey81@nevada.unlv.com

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\DA2A\T1. cpe301\DesignAssignments\DA2A\T1

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

REPEAT ; //blinking LED

DELAY_1: //Create a Delay of 60%DC

DELAY_2: //Create a Delay of 40%DC

DELAY_3: //Create a Delay of 1s

- DEVELOPED CODE OF TASK 1

;

; AssemblerApplication15.asm

;

; Created: 2019-03-09 오후 5:22:36

; Author : llje2

;

; Replace with your application code

.INCLUDE "M328PDEF.INC"				   //Include device file

.CSEG

.ORG	0x0000					   //Beginning of Program Memory

SBI DDRB,5 //TURN OFF THE D1LED

SBI PORTB,5

CBI DDRC,2 //make portc.2 input

SBI		DDRB, 2

REPEAT:	SBI		PORTB, 2	   //Sets pin (PB2) high

RCALL	DELAY_1		   //0.435SEC DELAY (60%DC)

CBI		PORTB, 2	   //Sets pin (PB2) low

RCALL	DELAY_2		   //0.29SEC DELAY (40%DC)

SBIC PINC,2    //IF BUTTON IS PRESSED,THEN NEXT INSTRU(INTERUPTION)

RJMP REPEAT   //Repeat continuously

CBI PORTB,2

RCALL	DELAY_3	 //1.25SEC delay

RCALL	DELAY_2

RJMP REPEAT

//-------------------------------------------------------------------------------------

DELAY_1:	    LDI		R16, 40			   //Create a Delay of 60%DC

LOOP1_1:	    LDI		R17, 87

LOOP2_1:	    LDI		R18, 109

LOOP3_1:        NOP

DEC		R18

BRNE	LOOP3_1

DEC		R17

BRNE	LOOP2_1

DEC		R16

BRNE	LOOP1_1

RET

DELAY_2:	    LDI		R16, 23			   //Create a Delay of 40%DC

LOOP1_2:	        LDI		R17, 58

LOOP2_2:	        LDI		R18, 72

LOOP3_2:	        NOP

DEC		R18

BRNE	LOOP3_2

DEC		R17

BRNE	LOOP2_2

DEC		R16

BRNE	LOOP1_2

RET

DELAY_3:	    LDI		R16, 80			   //Create a Delay of 1s

LOOP1_3:	    LDI		R17, 200

LOOP2_3:	    LDI		R18, 250

LOOP3_3:	    NOP

DEC		R18

BRNE	LOOP3_3

DEC		R17

BRNE	LOOP2_3

DEC		R16

BRNE	LOOP1_3

RET

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/fEA5JhAXYyA

- GITHUB LINK OF THIS DA

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\DA2A\T1. cpe301\DesignAssignments\DA2A\T2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

YEEUNLEE