CPE301 – SPRING 2019

Design Assignment 3A

Student Name: yeeunLee

Student #:2001614222

Student Email: leey81@nevada.unlv.com

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\DA3B

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

while (1) //Repeat continuously, display a temperature,

void adc_init(void) /* INIT ADC */

void read_adc(void) /* READ ADC PINS */

void USART_init( unsigned int ubrr ) /* INIT USART (RS-232)  */

void USART_tx_string( char *data ) /* SEND A STRING TO THE RS-232 */

ISR(TIMER0_COMPA_vect) // the interrupt service routine

- DEVELOPED CODE OF

/*

* GccApplication6.c

*

* Created: 2019-04-01 am6:51:27

* Author : llje2

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void read_adc(void);        // Function Declarations

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile unsigned int adc_temp;

char outs[20];

int extraTime=0; //to count 1second interrupt

int main(void)

{

DDRB = 0x01; // setting the LED as an output

TCCR0A = (1 << WGM01); // Set CTC Bit

OCR0A = 195; // number of ticks we need for our specific time

TIMSK0 = (1 << OCIE0A);

TCCR0B = (1 << CS02) | (1 << CS00); // use 1024 prescaler

sei(); // setting the interrupt

adc_init();             // Initialize the ADC

USART_init(BAUD_PRESCALLER);  // Initialize the USART

USART_tx_string("Connected!\r\n");    // we're alive!

_delay_ms(125);         // wait a bit

while(1)

{

read_adc();

snprintf(outs,sizeof(outs),"%3d\r\n", adc_temp);  // print it

USART_tx_string(outs);

_delay_ms(1000);        // wait a bit

}

}

/* INIT ADC */

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC5 (PC5 PIN28)

(1<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i = 4;

adc_temp = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp / 4;  // Average a few samples

}

/* INIT USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    //asynchronous 8 N 1

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

ISR(TIMER0_COMPA_vect) // the interrupt service routine(see list of vectors for all interrupt sources!!!!

{

extraTime++;

if(extraTime > 100)

{

PORTB ^= (1<<PORTB0); // toggle LED

extraTime = 0;

}

- }SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=VRhC_rOPMBw&feature=youtu.be

- GITHUB LINK OF THIS DA

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\DA3B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

YEEUNLEE