CPE301 – SPRING 2019

# MIDTERM 2

Student Name: yeeunLee

Student #:2001614222

Student Email: leey81@nevada.unlv.com

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\MIDTEM2

- COMPONENTS LIST OF CODE TASK1/A – DISPLAY COLOR SENSOR

List of Components used

Block diagram with pins used in the Atmega328P

void ADPS_Start_Loc()             // Initialize the ADPS

void Read_RawValue()          //READ VALUE OF the ADPS

USART_init(BAUD_PRESCALLER);  // Initialize the USART and initialize the pin number0

USART_tx_string("Connected!\r\n");/* SEND A STRING TO THE RS-232 */

while(1) IN MAIN()//Repeat continuously, current COLOR DATA.

- CODE OF TASK 1/A

/*

* color_sensor.c

*

* Created: 5/15/2019 3:36:25 AM

* Author : llje2

*/

#define F_CPU 16000000UL

#include <avr/io.h>		/* Include AVR std. library file */

#include <util/delay.h>		/* Include delay header file */

#include <inttypes.h>		/* Include integer type header file */

#include <stdlib.h>		/* Include standard library file */

#include <stdio.h>		/* Include standard I/O library file */

#include <stdbool.h>				/* Include standard boolean library */

#include "i2c_master.h"

#include "uart.h"	/* Include USART header file */

#include "APDS9960_def.h"

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

#include <util/delay.h>

int r,g,b,c, tmp = -11;

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

/* INIT USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    //asynchronous 8 N 1

}

void ADPS_Start_Loc()

{

i2c_start(0x72);	/* I2C start with device write address */

i2c_write(APDS9960_CDATAL);/* Write start location address from where to read */

i2c_start(0x73);/* I2C start with device read address */

}

void Read_RawValue()

{

ADPS_Start_Loc();   /* Read Gyro values */

c = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

r = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

g= (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

b= (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

i2c_stop();

}

int main(void)

{

char buffer[20];

uint8_t Connect_Status;

i2c_init();		/* Initialize I2C */

ADPS_Init();		/* Initialize Gyro */

USART_init(BAUD_PRESCALLER);  // Initialize the USART

USART_tx_string("Connected!\r\n");    // we're alive!

/* Replace with your application code */

while (1)

{

char _buffer[150];

Read_RawValue();

sprintf(buffer," clear: %d \t",c);

USART_tx_string(buffer);

sprintf(buffer, " Red: %d \t",r);

USART_tx_string(buffer);

sprintf(buffer," Green = %d \t",g);

USART_tx_string(buffer);

sprintf(buffer," Blue = %d \t",b);

USART_tx_string(buffer);

USART_tx_string("\n");

_delay_ms(5000);

}

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A) – UPLOAD THE VALUE ON THE THHINGSPEAK

List of Components used

Block diagram with pins used in the Atmega328P

void ADPS_Start_Loc()             // Initialize the ADPS

void Read_RawValue()          //READ VALUE OF the ADPS

USART_init(BAUD_PRESCALLER);  // Initialize the USART and initialize the pin number0

USART_tx_string("Connected!\r\n");/* SEND A STRING TO THE RS-232 */

while(1) IN MAIN()//Repeat continuously, current COLOR DATA.

`	// about the esp8266

ESP8266_WIFIMode(BOTH_STATION_AND_ACCESPOINT);/* 3 = Both (AP and STA) */

ESP8266_ConnectionMode(SINGLE);			/* 0 = Single; 1 = Multi */

ESP8266_ApplicationMode(NORMAL);

ESP8266_connected()

ESP8266_JoinAccessPoint(SSID, PASSWORD);

ESP8266_Start(0, DOMAIN, PORT);

ESP8266_Send(_buffer);

Read_Data(_buffer);

memset(_buffer, 0, 150);

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

/*

* color_sensor.c

*

* Created: 5/15/2019 3:36:25 AM

* Author : llje2

*/

#define F_CPU 16000000UL

#include <avr/io.h>		/* Include AVR std. library file */

#include <util/delay.h>		/* Include delay header file */

#include <inttypes.h>		/* Include integer type header file */

#include <stdlib.h>		/* Include standard library file */

#include <stdio.h>		/* Include standard I/O library file */

#include <stdbool.h>				/* Include standard boolean library */

#include "i2c_master.h"

#include "uart.h"	/* Include USART header file */

#include "APDS9960_def.h"

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

#include <util/delay.h>

int r,g,b,c, tmp = -11;

#define SREG    _SFR_IO8(0x3F)

#define DEFAULT_BUFFER_SIZE		160

#define DEFAULT_TIMEOUT			10000

/* Connection Mode */

#define SINGLE					0

#define MULTIPLE				1

/* Application Mode */

#define NORMAL					0

#define TRANSPERANT				1

/* Application Mode */

#define STATION							1

#define ACCESSPOINT						2

#define BOTH_STATION_AND_ACCESPOINT		3

/* Select Demo */

#define RECEIVE_DEMO				/* Define RECEIVE demo */

//#define SEND_DEMO					/* Define SEND demo */

/* Define Required fields shown below */

#define DOMAIN				"api.thingspeak.com"

#define PORT				"80"

#define API_WRITE_KEY		"PWJ8GZ3QXFQ0MXUM"

#define CHANNEL_ID			"751037"

#define SSID				"Leeyeeun"

#define PASSWORD			"02191919"

enum ESP8266_RESPONSE_STATUS{

ESP8266_RESPONSE_WAITING,

ESP8266_RESPONSE_FINISHED,

ESP8266_RESPONSE_TIMEOUT,

ESP8266_RESPONSE_BUFFER_FULL,

ESP8266_RESPONSE_STARTING,

ESP8266_RESPONSE_ERROR

};

enum ESP8266_CONNECT_STATUS {

ESP8266_CONNECTED_TO_AP,

ESP8266_CREATED_TRANSMISSION,

ESP8266_TRANSMISSION_DISCONNECTED,

ESP8266_NOT_CONNECTED_TO_AP,

ESP8266_CONNECT_UNKNOWN_ERROR

};

enum ESP8266_JOINAP_STATUS {

ESP8266_WIFI_CONNECTED,

ESP8266_CONNECTION_TIMEOUT,

ESP8266_WRONG_PASSWORD,

ESP8266_NOT_FOUND_TARGET_AP,

ESP8266_CONNECTION_FAILED,

ESP8266_JOIN_UNKNOWN_ERROR

};

int8_t Response_Status;

volatile int16_t Counter = 0, pointer = 0;

uint32_t TimeOut = 0;

char RESPONSE_BUFFER[DEFAULT_BUFFER_SIZE];

void Read_Response(char* _Expected_Response)

{

uint8_t EXPECTED_RESPONSE_LENGTH = strlen(_Expected_Response);

uint32_t TimeCount = 0, ResponseBufferLength;

char RECEIVED_CRLF_BUF[EXPECTED_RESPONSE_LENGTH];

while(1)

{

if(TimeCount >= (DEFAULT_TIMEOUT+TimeOut))

{

TimeOut = 0;

Response_Status = ESP8266_RESPONSE_TIMEOUT;

return;

}

if(Response_Status == ESP8266_RESPONSE_STARTING)

{

Response_Status = ESP8266_RESPONSE_WAITING;

}

ResponseBufferLength = strlen(RESPONSE_BUFFER);

if (ResponseBufferLength)

{

_delay_ms(1);

TimeCount++;

if (ResponseBufferLength==strlen(RESPONSE_BUFFER))

{

for (uint16_t i=0;i<ResponseBufferLength;i++)

{

memmove(RECEIVED_CRLF_BUF, RECEIVED_CRLF_BUF + 1, EXPECTED_RESPONSE_LENGTH-1);

RECEIVED_CRLF_BUF[EXPECTED_RESPONSE_LENGTH-1] = RESPONSE_BUFFER[i];

if(!strncmp(RECEIVED_CRLF_BUF, _Expected_Response, EXPECTED_RESPONSE_LENGTH))

{

TimeOut = 0;

Response_Status = ESP8266_RESPONSE_FINISHED;

return;

}

}

}

}

_delay_ms(1);

TimeCount++;

}

}

void ESP8266_Clear()

{

memset(RESPONSE_BUFFER,0,DEFAULT_BUFFER_SIZE);

Counter = 0;	pointer = 0;

}

void Start_Read_Response(char* _ExpectedResponse)

{

Response_Status = ESP8266_RESPONSE_STARTING;

do {

Read_Response(_ExpectedResponse);

} while(Response_Status == ESP8266_RESPONSE_WAITING);

}

void GetResponseBody(char* Response, uint16_t ResponseLength)

{

uint16_t i = 12;

char buffer[5];

while(Response[i] != '\r')

++i;

strncpy(buffer, Response + 12, (i - 12));

ResponseLength = atoi(buffer);

i += 2;

uint16_t tmp = strlen(Response) - i;

memcpy(Response, Response + i, tmp);

if(!strncmp(Response + tmp - 6, "\r\nOK\r\n", 6))

memset(Response + tmp - 6, 0, i + 6);

}

bool WaitForExpectedResponse(char* ExpectedResponse)

{

Start_Read_Response(ExpectedResponse);	/* First read response */

if((Response_Status != ESP8266_RESPONSE_TIMEOUT))

return true;							/* Return true for success */

return false;							/* Else return false */

}

bool SendATandExpectResponse(char* ATCommand, char* ExpectedResponse)

{

ESP8266_Clear();

USART_SendString(ATCommand);			/* Send AT command to ESP8266 */

USART_SendString("\r\n");

return WaitForExpectedResponse(ExpectedResponse);

}

bool ESP8266_ApplicationMode(uint8_t Mode)

{

char _atCommand[20];

memset(_atCommand, 0, 20);

sprintf(_atCommand, "AT+CIPMODE=%d", Mode);

_atCommand[19] = 0;

return SendATandExpectResponse(_atCommand, "\r\nOK\r\n");

}

bool ESP8266_ConnectionMode(uint8_t Mode)

{

char _atCommand[20];

memset(_atCommand, 0, 20);

sprintf(_atCommand, "AT+CIPMUX=%d", Mode);

_atCommand[19] = 0;

return SendATandExpectResponse(_atCommand, "\r\nOK\r\n");

}

bool ESP8266_Begin()

{

for (uint8_t i=0;i<5;i++)

{

if(SendATandExpectResponse("ATE0","\r\nOK\r\n")||SendATandExpectResponse("AT","\r\nOK\r\n"))

return true;

}

return false;

}

bool ESP8266_Close()

{

return SendATandExpectResponse("AT+CIPCLOSE=1", "\r\nOK\r\n");

}

bool ESP8266_WIFIMode(uint8_t _mode)

{

char _atCommand[20];

memset(_atCommand, 0, 20);

sprintf(_atCommand, "AT+CWMODE=%d", _mode);

_atCommand[19] = 0;

return SendATandExpectResponse(_atCommand, "\r\nOK\r\n");

}

uint8_t ESP8266_JoinAccessPoint(char* _SSID, char* _PASSWORD)

{

char _atCommand[60];

memset(_atCommand, 0, 60);

sprintf(_atCommand, "AT+CWJAP=\"%s\",\"%s\"", _SSID, _PASSWORD);

_atCommand[59] = 0;

if(SendATandExpectResponse(_atCommand, "\r\nWIFI CONNECTED\r\n"))

return ESP8266_WIFI_CONNECTED;

else{

if(strstr(RESPONSE_BUFFER, "+CWJAP:1"))

return ESP8266_CONNECTION_TIMEOUT;

else if(strstr(RESPONSE_BUFFER, "+CWJAP:2"))

return ESP8266_WRONG_PASSWORD;

else if(strstr(RESPONSE_BUFFER, "+CWJAP:3"))

return ESP8266_NOT_FOUND_TARGET_AP;

else if(strstr(RESPONSE_BUFFER, "+CWJAP:4"))

return ESP8266_CONNECTION_FAILED;

else

return ESP8266_JOIN_UNKNOWN_ERROR;

}

}

uint8_t ESP8266_connected()

{

SendATandExpectResponse("AT+CIPSTATUS", "\r\nOK\r\n");

if(strstr(RESPONSE_BUFFER, "STATUS:2"))

return ESP8266_CONNECTED_TO_AP;

else if(strstr(RESPONSE_BUFFER, "STATUS:3"))

return ESP8266_CREATED_TRANSMISSION;

else if(strstr(RESPONSE_BUFFER, "STATUS:4"))

return ESP8266_TRANSMISSION_DISCONNECTED;

else if(strstr(RESPONSE_BUFFER, "STATUS:5"))

return ESP8266_NOT_CONNECTED_TO_AP;

else

return ESP8266_CONNECT_UNKNOWN_ERROR;

}

uint8_t ESP8266_Start(uint8_t _ConnectionNumber, char* Domain, char* Port)

{

bool _startResponse;

char _atCommand[60];

memset(_atCommand, 0, 60);

_atCommand[59] = 0;

if(SendATandExpectResponse("AT+CIPMUX?", "CIPMUX:0"))

sprintf(_atCommand, "AT+CIPSTART=\"TCP\",\"%s\",%s", Domain, Port);

else

sprintf(_atCommand, "AT+CIPSTART=\"%d\",\"TCP\",\"%s\",%s", _ConnectionNumber, Domain, Port);

_startResponse = SendATandExpectResponse(_atCommand, "CONNECT\r\n");

if(!_startResponse)

{

if(Response_Status == ESP8266_RESPONSE_TIMEOUT)

return ESP8266_RESPONSE_TIMEOUT;

return ESP8266_RESPONSE_ERROR;

}

return ESP8266_RESPONSE_FINISHED;

}

uint8_t ESP8266_Send(char* Data)

{

char _atCommand[20];

memset(_atCommand, 0, 20);

sprintf(_atCommand, "AT+CIPSEND=%d", (strlen(Data)+2));

_atCommand[19] = 0;

SendATandExpectResponse(_atCommand, "\r\nOK\r\n>");

if(!SendATandExpectResponse(Data, "\r\nSEND OK\r\n"))

{

if(Response_Status == ESP8266_RESPONSE_TIMEOUT)

return ESP8266_RESPONSE_TIMEOUT;

return ESP8266_RESPONSE_ERROR;

}

return ESP8266_RESPONSE_FINISHED;

}

int16_t ESP8266_DataAvailable()

{

return (Counter - pointer);

}

uint8_t ESP8266_DataRead()

{

if(pointer < Counter)

return RESPONSE_BUFFER[pointer++];

else{

ESP8266_Clear();

return 0;

}

}

uint16_t Read_Data(char* _buffer)

{

uint16_t len = 0;

_delay_ms(100);

while(ESP8266_DataAvailable() > 0)

_buffer[len++] = ESP8266_DataRead();

return len;

}

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

/* INIT USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    //asynchronous 8 N 1

}

void ADPS_Init()		/* Gyro initialization function */

{

//_delay_ms(150);		/* Power up time >100ms */

tmp = i2c_start(0x72);	/* Start with device write address */

tmp = i2c_write(APDS9960_ENABLE);	/* Write to sample rate register */

tmp = i2c_write(0x03);	/* 1KHz sample rate */

i2c_stop();

tmp = i2c_start(0x72);

tmp = i2c_write(APDS9960_ATIME);	/* Write to power management register */

i2c_write(0xdb);

i2c_stop();

tmp = i2c_start(0x72);

tmp = i2c_write(APDS9960_CONTROL);	/* Write to Configuration register */

i2c_write (0x01);

i2c_stop();

}

void ADPS_Start_Loc()

{

i2c_start(0x72);	/* I2C start with device write address */

i2c_write(APDS9960_CDATAL);/* Write start location address from where to read */

i2c_start(0x73);/* I2C start with device read address */

}

void Read_RawValue()

{

ADPS_Start_Loc();   /* Read Gyro values */

c = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

r = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

g= (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

b= (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

i2c_stop();

}

int main(void)

{

char buffer[20];

uint8_t Connect_Status;

i2c_init();		/* Initialize I2C */

ADPS_Init();		/* Initialize Gyro */

USART_init(BAUD_PRESCALLER);  // Initialize the USART

USART_tx_string("Connected!\r\n");    // we're alive!

/* Replace with your application code */

ESP8266_WIFIMode(STATION); //3 = Both (AP and STA) */

ESP8266_ConnectionMode(SINGLE);			/* 0 = Single; 1 = Multi */

ESP8266_ApplicationMode(NORMAL);		/* 0 = Normal Mode; 1 = Transperant Mode */

if(ESP8266_connected() == ESP8266_NOT_CONNECTED_TO_AP)

ESP8266_JoinAccessPoint(SSID, PASSWORD);

ESP8266_Start(0, DOMAIN, PORT);

while (1)

{

char _buffer[150];

Read_RawValue();

Connect_Status = ESP8266_connected();

if(Connect_Status == ESP8266_NOT_CONNECTED_TO_AP)

ESP8266_JoinAccessPoint(SSID, PASSWORD);

if(Connect_Status == ESP8266_TRANSMISSION_DISCONNECTED)

ESP8266_Start(0, DOMAIN, PORT);

sprintf(buffer," clear: %d \t",c);

USART_tx_string(buffer);

sprintf(_buffer, "GET /update?api_key=%s&field1=%3d", API_WRITE_KEY, c);

ESP8266_Send(_buffer);

sprintf(buffer, " Red: %d \t",r);

USART_tx_string(buffer);

sprintf(_buffer, "GET /update?api_key=%s&field2=%3d", API_WRITE_KEY, r);

ESP8266_Send(_buffer);

sprintf(buffer," Green = %d \t",g);

USART_tx_string(buffer);

sprintf(_buffer, "GET /update?api_key=%s&field3=%3d", API_WRITE_KEY, g);

ESP8266_Send(_buffer);

sprintf(buffer," Blue = %d \t",b);

USART_tx_string(buffer);

sprintf(_buffer, "GET /update?api_key=%s&field4=%3d", API_WRITE_KEY, b);

ESP8266_Send(_buffer);

USART_tx_string("\n");

_delay_ms(5000);

}

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (THINGSPEAK)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/BhCjwT7UXCk

- GITHUB LINK OF THIS DA

Primary Github address: https://github.com/yeeun219/submission_da.git

Directory: cpe301\DesignAssignments\MIDTERM2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT